Pumps Refresher – Drafting Operations

This month's drill was prepared by
MFRI Field Instructor Fran Jester

INSTRUCTOR GUIDE

Time required: 3 Hours

Teaching/Learning Materials:  Two fully equipped pumpers

Portable master stream devices

Barrel and floating type suction strainers

Pitot gauge

References: Pump Operator Student Manual, Maryland Fire and Rescue Institute; Pumping Apparatus Driver/Operator Handbook, IFSTA, 2nd Edition

Motivation:

Student Performance Objective: The student will demonstrate an understanding of the principles of drafting from a static water source and provide an adequate flow of water to a portable master stream device during practical evolutions, to the satisfaction of the instructor

Enabling Objectives

  - EO 1-1 Describe the theory of drafting.

  - EO 1-2 Describe the criteria for a good drafting location.

  - EO 1-3 List common problems that can occur during drafting, their indicators and corrective actions to take.

  - EO 1-4 Describe the basic operating practices when operating a pumper from draft.

  - EO 1-5 Demonstrate proper drafting procedures during practical evolutions

Overview:

- Principles of drafting

- Considerations for a good draft site

- Common problems during drafting operations

- Drafting operating practices

- Practical application

I.	Principles of Drafting (1-1)

  - Based on the principle of creating a negative pressure (vacuum) inside the fire pump and allowing atmospheric pressure on the water surface to force water up the suction tube into the fire pump.

    - Perfect vacuum provides 14.7 psi of negative pressure at sea level or 30 inches of vacuum.

    - A fire apparatus primer must be able to develop 22 inches of vacuum in 30 seconds.

    - 22 inches of vacuum is equivalent to 10.5 psi of negative pressure and will lift water approximately 24 feet.

    - The theoretical maximum lift with a perfect vacuum at sea level is 33.9 feet. The maximum practical lift for fireground operations is 20 feet.

    - Fire apparatus pumps are designed to supply their rated capacity at a lift of no higher than 10 feet.

    - The size of the suction hose / hard sleeve affects the maximum flow for a given lift.

    - The vacuum reading on the compound gauge is the basis for estimating the ability of the pump to supply additional GPM. Maximum flow from a fire pump at draft would be 22 inches of vacuum.

- Criteria for a Good Drafting Location (1-2)

  - Amount of Water Available

  - Depth of Water source / Replenishment Rate

  - Site Accessibility (year round)

    - Stability of Ground

    - Height of Lift from Water Surface

  - Quality of Water

    - Sediment

    - Salt Content

    - Debris

    - Chemicals

- Common Problems Encountered While Drafting (1-3)

  - Watch Your Vacuum Gauge

  - Unable to Obtain a Vacuum / loss of Prime

    - Air Leaks in Pump or Suction Hose

      - Check for loose hose connections

      - Check for open drain valves

  - Cavitation of the Pump / High Vacuum Reading

    - Blockage in the Strainer or Suction Hose

      - Check strainer for debris

      - Strainer in mud

    - Excessive Flow for Height of Lift

      - Reduce flow

      - Reduce amount of hose lines

- Basic Operating Practices for Drafting (1-4)

  - Position Pumper as Near as Possible to Water Source

  - Apply Parking Break / Chock Wheels

  - Connect Strainer and Hard Suction Hose(s) with Airtight Connections

    - Barrel Strainer

      - Use ladder and/or rope to keep it off of the bottom and out of the mud

      - Need at least 18” of water above and below strainer to prevent whirl pooling

2.  Floating Dock Strainers should be used for Shallow Water Operations

  - Engage Fire Pump / Put Vehicle Transmission in Proper Gear

  - Two Stage Pumps should have Transfer Valve put in Parallel (volume) or Series (Pressure) Position

  - Engage Primer Valve until Pump is Full of Water (approximately 15 – 20 seconds)

  - Increase Throttle Slowly until Pressure Gauge Reads at least 50 psi

  - Open Discharge Valve Slowly – Disengage Primer

  - Discharge Water back into Water Source to keep Pump Cool when not flowing Water

  - Set Relief Valve or Pressure Governor if appropriate

  - Watch Vacuum Gauge during Drafting Operation for possible Problems

- Practical Application (1-5)

  - Utilizing Apparatus Provided, set up Drafting Evolutions from Various Static Water Sources as Appropriate:

    - Stream

    - Lake

    - Pond

    - Dry Hydrant

    - Portable Tanks

    - Underground Tanks (cisterns)

  - Use Portable Master Stream Device for Fire Flow

    - Students should check Nozzle Pressure with Pitot Gauge to Maintain Desired Flow

  - Rotate each of the Students through the Drafting Process using various Apparatus to insure Proficiency in this sometimes forgotten Skill.

Summary:

Student Performance Objective: The student will demonstrate an understanding of the principles of drafting from a static water source and provide and adequate flow of water to a portable master stream device during practical evolutions, to the satisfaction of the instructor

Review:

- Principles of drafting

- Considerations for a good draft site

- Common problems during drafting operations

- Drafting operating practices

- Practical application